1.2	What is reliability of computers?

Reliability is an attribute of any computer-related component -- software, hardware or a network, for example -- that consistently performs according to its specifications. It has long been considered one of three related attributes that must be considered when making, buying or using a computer product or component.

Reliability is a key consideration when buying computer products. Computer system and component manufacturers and software developers often highlight it as an attribute of their products. The more upgrades and updates a product has undergone, the more likely it is that problems have been addressed, making it more reliable.

Maintainability: The probability that a failed item will be restored to its satisfactory operational state.

1.3	How does reliability work?

Reliability takes many forms. A reliable computer exhibits dependability and has high uptime and low downtime and system failure rates. System reliability can be achieved using techniques such as redundancy.

Redundant systems have multiple copies of critical components available. That way, if a primary component fails, a backup can be used. Redundant systems also have fault tolerance, which is the ability of a computing device to absorb a disruptive event, such as a power outage, and quickly resume operation and output.

In storage, RAID 1's disk mirroring provides redundancy.

Reliability, availability and serviceability are important aspects to design into a system. In computer science and computer systems theory, a reliable product is free of technical errors; in practice, however, vendors frequently express a product's reliability quotient as a percentage.

Evolutionary products -- those that providers have evolved through numerous versions over time -- are thought to become increasingly reliable as bugs are eliminated in each subsequent release. For example, IBM's z/OS, an operating system for its S/390 server series, has a reputation for being reliable. It evolved from a long line of earlier Multiple Virtual Storage and OS/390 versions. Eight ways computer systems are tested.

1.4	What metrics can demonstrate reliability?

Various metrics are used to measure computing system reliability and the probability of failure, including the following:

System availability

This is the ratio of a system's actual operating time divided by the total amount of time it should be available, which is its uptime plus its downtime. The closer to 1 that number is, the better its availability and the more reliable it is because that would be equivalent to 100% availability.

System availability = Total uptime ÷ (Total uptime + Total downtime)

For example, a server runs for 500 hours before having an issue that takes 10 hours to repair. It then runs another 970 hours before having another issue that takes 20 hours to repair. Its system availability is the total operating time of 1,470 hours divided by the total time the server should have been available, which was 1,500 hours. That equals 0.98, meaning the server was available 98% of the time.

Mean time between failures (MTBF)

This is the average amount of time between system or component failures. To calculate MTBF, divide a system's total operating time by the number of downtime incidents. This metric is used for repairable systems or components.

MTBF = Total uptime ÷ Number of downtime incidents

The MTBF for the server discussed above would be the total uptime of 1,470 hours divided by two incidents, making the MTBF 735 hours.

Mean time to repair (MTTR)

This measure how fast a disabled or failed component can be returned to normal operations. MTTR is calculated by taking the total downtime a system experiences in a given time period and dividing it by the number of downtime incidents. The faster a system or component can be repaired, the lower the MTTR.

MTTR = Total downtime ÷ Number of downtime incidents

In the server example, the MTTR would be the 30 hours of downtime divided by the two incidents, equaling 15 hours.

Mean time to failure (MTTF)

This is the average period of time a non-repairable system or component will run before it fails. This is only used for systems or parts that can't or aren't going to be repaired. In most organizations, it wouldn't apply to the server example; it's more likely to apply to a part such as a light bulb that would be replaced rather than repaired. It's calculated by adding the total operating time before failure of several light bulbs and dividing that total by the number of light bulbs being assessed.

MTTF = Total hours of operation ÷ Total number of assets in use

1.5	How is reliability tested and verified?

Many diagnostic and testing tools are available to measure the reliability of systems and networks. Checking manufacturer reports on a device's performance can provide good information on its reliability. User and third-party assessments are also a good way to get insight.

Manufacturers continue to do reliability testing and validation even after a system or device goes into production. IT system and software engineering teams also do ongoing quality management activities. Third-party companies experienced in reliability testing can also be employed for these assessments.

During software development, a key part of the process is to regularly test the software to make sure it performs as designed and is reliable. Running applications continuously for a period is one test. Another is to send many inquiries to the software to see how it reacts.

Black box testing is a software testing methodology where the tester doesn't know how the system is generating responses to the test requests.

1.6	What organization sets computer reliability standards?

The Institute of Electrical and Electronics Engineers, or IEEE, Reliability Society is an organization devoted to reliability in engineering. It promotes a systematic approach to design that fosters reliability in engineering, maintenance and analysis.

The Reliability Society encourages collaborative effort and information sharing among members. The various industries represented in the group include aerospace, transportation systems, medical electronics, computers, telecommunications and other areas of engineering.

Various associations and governmental bodies provide input into product reliability in specific industries. For example, the International Automotive Task Force plays such a role in the automotive industry.

1.7	Tips for ensuring technology reliability

The following are ways to ensure systems, networks and software perform as reliably as possible:

- Get IT leadership support for reliability programs.

- Allocate sufficient funding for reliability testing in budgets.

- Set up schedules for regular testing.

- Identify resources to keep systems functioning at optimal levels, such as spare parts, backup power systems, and backup of data and software.

- Document all reliability testing activities and compare previous reports with current performance to identify opportunities for improvement.

- Conduct periodic risk assessments to identify potential threats and vulnerabilities that could affect system reliability.

- Ensure disaster recovery plans contain information on how to recover and restart critical systems in an emergency.

1.8	THE BATHTUB CURVE

The bathtub curve is a particular shape of a failure rate graph. This graph is used in reliability engineering and deterioration modeling. The 'bathtub' refers to the shape of a line that curves up at both ends, similar in shape to a bathtub as it is represented in figure 1.1. The bathtub curve has 3 regions:

- The first region has a decreasing failure rate due to early failures.

- The middle region is a constant failure rate due to random failures.

- The last region is an increasing failure rate due to wear-out failures.

Not all products exhibit a bathtub curve failure rate. A product is said to follow the bathtub curve if in the early life of a product, the failure rate decreases as defective products are identified and discarded, and early sources of potential failure such as manufacturing defects or damage during transit are detected. In the mid-life of a product the failure rate is constant. In the later life of the product, the failure rate increases due to wearout. Many electronic consumer product life cycles follow the bathtub curve. It is difficult to know where a product is along the bathtub curve, or even if the bathtub curve is applicable to a certain product without large amounts of products in use and associated failure rate data.

If products are retired early or have decreased usage near their end of life, the product may show fewer failures per unit calendar time (but not per unit use time) than the bathtub curve predicts.

In reliability engineering, the cumulative distribution function corresponding to a bathtub curve may be analysed using a Weibull chart or in a reliability contour map.

Figure 1.1: The bathtub curve failure rate graph

CHAPTER TWO

COMPUTER HARDWARE SYSTEM

Computer hardware includes the physical parts of a computer, such as the case, central processing unit (CPU), random access memory (RAM), monitor, mouse, keyboard, computer data storage, graphics card, sound card, speakers and motherboard.

By contrast, software is the set of instructions that can be stored and run by hardware. Hardware is so-termed because it is hard or rigid with respect to changes, whereas software is soft because it is easy to change.

Hardware is typically directed by the software to execute any command or instruction. A combination of hardware and software forms a usable computing system, although other systems exist with only hardware.

- CASE: The computer case encloses most of the components of the system. It provides mechanical support and protection for internal elements such as the motherboard, disk drives, and power supplies, and controls and directs the flow of cooling air over internal components. The case is also part of the system to control electromagnetic interference radiated by the computer and protects internal parts from electrostatic discharge.

- POWER SUPPLY: A power supply unit (PSU) converts alternating current (AC) electric power to low-voltage direct current (DC) power for the computer. Laptops can run on built-in rechargeable battery.

- MOTHERBOARD is the main component of a computer. It is a board with integrated circuitry that connects the other parts of the computer including the CPU, the RAM, the disk drives (CD, DVD, hard disk, or any others) as well as any peripherals connected via the ports or the expansion slots. The integrated circuit (IC) chips in a computer typically contain billions of tiny metal–oxide–semiconductor field-effect transistors (MOSFETs). Components directly attached to or to part of the motherboard include: CPU, CHIPSET, RAM, ROM, BUSES, CMOS BATTERY which powers the cmos memory for date and time in the BIOS chip.

- EXPANSION CARDS: An expansion card in computing is a printed circuit board that can be inserted into an expansion slot of a computer motherboard or backplane to add functionality to a computer system via the expansion bus. Expansion cards can be used to obtain or expand on features not offered by the motherboard.

- STORAGE DEVICES: A storage device is any computing hardware and digital media that is used for storing, porting and extracting data files and objects. It can hold and store information both temporarily and permanently and can be internal or external to a computer, server or any similar computing device. Data storage is a core function and fundamental component of computers. Dedicated storage devices include RAIDs and tape libraries.

HARDWARE UPGRADE

When using computer hardware, an upgrade means adding new or additional hardware to a computer that improves its performance, increases its capacity, or adds new features. For example, a user could perform a hardware upgrade to replace the hard drive with a faster one or a solid state drive (SSD) to get a boost in performance. The user may also install more Random Access Memory (RAM) so the computer can store additional temporary data, or retrieve such data at a faster rate. The user may add a USB 3.0 expansion card to fully use USB 3.0 devices, or could upgrade the Graphics Processing Unit (GPU) for cleaner, more advanced graphics, or more monitors. Performing such hardware upgrades may be necessary for aged computers to meet a new, or updated program's system requirements.

In large organizations, hardware upgrades are handled by administrators who are also in charge of keeping networks running smoothly. They replace network devices like servers, routers and storage devices based on new demands and capacities.

Toxic computer components

The central processing unit contains many toxic materials. It contains lead and chromium in the metal plates. Resistors, semiconductors, infrared detectors, stabilizers, cables, and wires contain cadmium. The circuit boards in a computer contain mercury, and chromium.] When these types of materials, and chemicals are disposed improperly will become hazardous for the environment.

Recycling of computer hardware is considered environmentally friendly because it prevents hazardous waste, including heavy metals and carcinogens, from entering the atmosphere, landfill or waterways. While electronics consist a small fraction of total waste generated, they are far more dangerous.

CHAPTER THREE

COMMUNICATION EQUIPMENTS

A communication device is a hardware device capable of transmitting an analog or digital signal over the telephone, other communication wire, or wirelessly.

A classic example of a communication device is a computer modem, which converts a computer's digital information to an analog signal for transmission over a telephone line. Similarly, a modem receives analog signals, and converts them to digital, for processing by the computer. This process is called modulation/demodulation, from which the modem gets its name.

Other examples of communication devices include a NIC (network interface card), Wi-Fi devices, and access points.

Communication device examples

Below is a full listing of all the different types of communications devices you may encounter when dealing with a computer.

- Bluetooth devices

- Infrared devices

- Modem (over phone line)

- Network card (using Ethernet)

- Smartphone

- Wi-Fi devices (using a Wi-Fi router); examples shown below.

Communication device errors

Any time a communication device encounters problems communicating with another device, you may encounter a communication error. Below is a list of general steps that can be verified when this error is encountered.

- If your communication device uses a wire, make sure it is firmly connected.

- Make sure the proper drivers are installed and that no errors or conflicts are occurring with the driver, and that the device is detected.

- If your communication device requires settings, make sure they're correct. For example, a network card requires the proper IP (Internet Protocol), DNS (domain name system), and Subnet settings to establish a connection with the network, and be able to find its route to other network devices.

Why do computers need communication devices?

A computer can work fine without a communication device. However, for a computer to communicate with other computers, they need a communication device. For example, for your computer to connect to the Internet to view this web page, it needs a communication device. Without a communication device, you'd have to use a sneaker net to transfer or share data between computers.

BASIC ELECTRONICS COMPONENTS

Basic Electronic Components are electronic devices or parts usually packaged in a discrete form with two or more connecting leads or metallic pads. These devices are intended to be connected together, usually by soldering to a Printed Circuit Board (PCB), to create an electronic circuit with a particular function (for example an amplifier, radio receiver, oscillator, wireless).

Some of the Basic Electronic Components are: resistor, capacitor, transistor, diode, operational amplifier, resistor array, logic gate etc.

There are two types: (1) Passive and (2) Active Components

1. Passive Components

These components are those that do not have gain or directionality. They are also called Electrical elements or electrical components.

Example: Resistors, Capacitors, Diodes, Inductors.

2. Active Components

These components are those that have gain or directionality.

Example: Transistors, Integrated Circuits or ICs, Logic Gates.

Function of Basic Electronic Components

- Terminals and Connectors: Components to make electrical connection.

- Resistors: Components used to resist current.

- Switches: Components that may be made to either conduct (closed) or not (open).

- Capacitors: Components that store electrical charge in an electrical field.

- Magnetic or Inductive Components: These are Electrical components that use magnetism.

- Network Components: Components that use more than 1 type of Passive Component.

- Piezoelectric devices, crystals, resonators: Passive components that use piezoelectric. effect.

- Semiconductors: Electronic control parts with no moving parts.

- Diodes: Components that conduct electricity in only one direction.

- Transistors: A semiconductor device capable of amplification.

- Integrated Circuits or ICs: A microelectronic computer circuit incorporated into a chip or semiconductor; a whole system rather than a single component.

CHAPTER FOUR

BASIC COMPUTER MAINTAINANCE

Computer maintenance is the practice of keeping computers in a good state of repair. A computer containing accumulated dust and debris may not run properly.

FOUR TYPE OF MAINTAINANCE

- CORRECTIVE MAINTENANCE: Maintenance is carried out following detection of an anomaly and aimed at restoring normal operating conditions. This approach is based on the firm belief that the costs sustained for downtime and repair in case of fault are lower than the investment required for a maintenance program. This strategy may be cost-effective until catastrophic faults occur.

- PREVENTIVE MAINTENANCE: Maintenance carried out at predetermined intervals or according to prescribed criteria, aimed at reducing the failure risk or performance degradation of the equipment.
The maintenance cycles are planned according to the need to take the device out of service. The incidence of operating faults is reduced.

- RISK BASED MAINTENANCE: Maintenance carried out by integrating analysis, measurement and periodic test activities to standard preventive maintenance. The gathered information is viewed in the context of the environmental, operation and process condition of the equipment in the system. The aim is to perform the asset condition and risk assessment and define the appropriate maintenance program.
 All equipment displaying abnormal values is refurbished or replaced. In this way it is possible to extend the useful life and guarantee over time high levels of reliability, safety and efficiency of the plant.

- CONDITION BASED MAINTENANCE: Maintenance based on the equipment performance monitoring and the control of the corrective actions taken as a result.
The real actual equipment condition is continuously assessed by the on-line detection of significant working device parameters and their automatic comparison with average values and performance. Maintenance is carried out when certain indicators give the signaling that the equipment is deteriorating and the failure probability is increasing.

This strategy, in the long term, allows reducing drastically the costs associated with maintenance, thereby minimizing the occurrence of serious faults and optimizing the available economic resources management.

# FAULTY TROUBLESHOOTING TECHNIQUE

# Troubleshooting is a systematic approach to solving a problem. The goal of troubleshooting is to determine why something does not work as expected and explain how to resolve it.

The Five Steps of Troubleshooting

You're systematic approach to troubleshooting should involve five basic steps;

- Information Gathering

- Analysis and Planning

- Implementation of a solution

- Assessment of the effectiveness of the solution

- Documentation of the incident

Below we will address these steps individually.

- Information Gathering: Before we can determine how to address a problem--or even assess what the problem is--we must gather information. Gathering information can be particularly challenging when the problem manifests itself at the client side. You might have to formulate your questions carefully in order to get meaningful information. Log files contain great information you should consult during the data-gathering stage. Both the Windows logs and the AIRA logs should be consulted. Also check to see if all Services are running properly.

- Analysis Once we have gathered the data (including attempting to reproduce the problem), it's time to analyze the data.  The primary task in this phase is to look for patterns. An important part of the analysis phase involves prioritizing. This includes prioritizing the problems, if there are multiple problems. Performance problems are generally less urgent than access problems.

- Solution Implementation Although there could be several possible solutions to a problem, you should always implement one change at a time. Assess the results of that change before trying something else. This will save you time in the long run.

- Assessment  It is vital that you assess the results of your actions and determine whether the "fix" worked, whether it was a temporary work around, or whether it caused additional problems.

- Documentation After completing your assessment, you should develop a summarization of the problem, which should include the reported and observed symptoms, the corrective actions taken, and the results of those actions.

# CHAPTER FIVE

# QUALITY OF SERVICE (QOS)

What is quality of service?

Quality of service (QoS) refers to any technology that manages data traffic to reduce packet loss, latency and jitter on a network. QoS controls and manages network resources by setting priorities for specific types of data on the network.

Enterprise networks need to provide predictable and measurable services as applications -- such as voice, video and delay-sensitive data -- to traverse a network. Organizations use QoS to meet the traffic requirements of sensitive applications, such as real-time voice and video, and to prevent the degradation of quality caused by packet loss, delay and jitter.

Organizations can reach a QoS by using certain tools and techniques, such as jitter buffer and traffic shaping. For many organizations, QoS is included in the service-level agreement (SLA) with their network service provider to guarantee a certain level of network performance.

Class of service (CoS) is sometimes used interchangeably with QoS, though they are slightly different. CoS technology does not specifically guarantee a level of service in terms of bandwidth, and it approaches traffic control from a less-granular perspective. In many cases, whether someone refers to CoS or QoS, they are likely talking about the same concept.

QoS parameters

Organizations can measure QoS quantitatively by using several parameters:

- Packet loss. This happens when network links become congested and routers and switches start dropping packets. When packets are dropped during real-time communication, such as in voice or video calls, these sessions can experience jitter and gaps in speech. Packets can be dropped when a queue, or line of packets waiting to be sent, overflows.

- Jitter. This is the result of network congestion, timing drift and route changes. Too much jitter can degrade the quality of voice and video communication.

- Latency. This is the time it takes a packet to travel from its source to its destination. Latency should be as close to zero as possible. If a voice over IP call has a high amount of latency, users can experience echo and overlapping audio.

- Bandwidth. This is the capacity of a network communications link to transmit the maximum amount of data from one point to another in a given amount of time. QoS optimizes the network performance by managing bandwidth and giving high priority applications with stricter performance requirements more resources than others.

- Mean opinion score. This is a metric to rate voice quality that uses a five-point scale, with a five indicating the highest quality.

Why is QoS important?

Without QoS, network data can become disorganized, clogging networks to the point where performance degrades or, in certain cases, the network shuts down completely.

QoS is important because enterprises need to provide stable services for employees and customers to use. Quality of service determines quality of experience (QoE). If the services an organization provides are not reliable, customer and employee relationships may be put at risk.

Additionally, data integrity and security are more likely to be compromised in a company with a poor QoS. In general, employees and customers depend on communication services to do their jobs. When QoS suffers, quality of work and QoE suffer as well.

How does QoS work?

When organizations use their networks to send information back and forth between endpoints on the network, the information or data is formatted into packets. Packets are the way computers organize information to be transferred over a network, like how a person might package items to be sent through the physical mail system.

QoS tools take on the responsibility of prioritizing packets to get the most out of the finite amount of bandwidth on their network. In other words, the network can only transport so much information in a certain amount of time. QoS tools prioritize packets in a way that ensures that bandwidth is used to provide the best internet service possible in that fixed amount of time.

For example, packets pertaining to a video call would be prioritized over packets pertaining to an email download. This is because a video call is a more synchronous form of communication than an email -- video needs to happen in real time, whereas emailing is not necessarily time sensitive. If a packet is dropped or delayed during a video chat, the end user may experience jitter or latency in the chat. If packets are dropped or delayed in the emailing process, they can still be sent after, and the end user will not experience any lapse in service. They will only receive the email when all packets have been assembled, whereas someone streaming video sees the packets as they arrive.

A QoS tool looks at packet headers to prioritize packets. Packet headers are bits of information that tell the tool and other network components what the packet contains, where it is going (the IP address of its destination) and what it will be used for.

The QoS tool can also read the packet header, determine a packet is related to video streaming and prioritize it over packets that are less time sensitive. A packet header can be thought of as the mailing and return addresses on a physical package. The QoS tool can alter a portion of the packet header to specify priority.

What are the benefits of QoS?

The main benefit of QoS is that it ensures the availability of an organization's network and the applications that run on that network. It provides the safe, efficient transfer of data over that network. QoS also allows organizations to use their existing bandwidth more efficiently, instead of upgrading network infrastructure to expand bandwidth.

More specific benefits include the following:

- Mission-critical applications have access to the resources they require.

- Administrators can manage traffic better.

- Organizations can reduce costs by eliminating the need to purchase new network infrastructure.

- User experience is improved.

Implementing QoS

When implementing QoS tools and strategies, organizations should follow these steps:

- Planning. The organization should gain an understanding of each department's service needs and requirements, choose a fitting model and cultivate buy-in from stakeholders.

- Design. The organization should then take note of all significant software and hardware changes and apply the chosen QoS model to the specifics of its network architecture.

- Testing. The organization should test QoS settings and policies in a safe, controlled testing environment where bugs can be worked out.

- Deployment. Policies should be rolled out in phases. An organization may choose to roll out policies by network segment or by separate QoS function (what each policy does).

- Monitoring and analysis. Policies should be adjusted to improve performance according to performance data.

Three models exist to implement QoS. They are Best Effort, Integrated Services and Differentiated Services:

- Best Effort. A QoS model where all the packets receive the same priority and there is no guaranteed delivery of packets. Best Effort is applied when networks have not configured QoS policies or when the infrastructure does not support QoS.

- Integrated Services (IntServ). A QoS model that reserves bandwidth along a specific path on the network. Applications ask the network for resource reservation, and network devices monitor the flow of packets to make sure network resources can accept the packets.

- Differentiated Services (DiffServ). A QoS model where network elements, such as routers and switches, are configured to service multiple classes of traffic with different priorities. Network traffic must be divided into classes based on a company's configuration.

Implementing IntServ requires IntServ-capable routers and uses the Resource Reservation Protocol for network resource reservation. IntServ has limited scalability and high consumption of network resources.

Voice traffic can be assigned a higher priority than other types of traffic. Packets are assigned priorities using Differentiated Services Code Point for classification. DiffServ also uses per-hop behavior to apply QoS techniques, such as queuing and prioritization, to packets.

Network architecture also affects how an organization implements QoS. A Multiprotocol Label Switching (MPLS) network includes a private link that offers end-to-end QoS along a single path. SLAs for MPLS specify bandwidth, QoS, latency and uptime. However, an MPLS can be expensive for organizations.

Software-defined WAN (SD-WAN) uses multiple connectivity types, including MPLS and broadband. SD-WAN monitors the state of current network connections for performance issues and uses its multiple connectivity types to fail over, based on the state of the connection. For example, if packet loss exceeds a certain level on one connection, SD-WAN capabilities will look for an alternative connection.

QoS mechanisms

Certain QoS mechanisms can manage data traffic quality and maintain the QoS requirements specified in SLAs. QoS mechanisms fall under specific categories, depending on the roles they play in managing the network.

- Classification and marking. These tools differentiate between applications and sort packets into different traffic types. Marking indicates each packet as a member of a network class, which helps devices on the network recognize the packet's class. Classification and marking are implemented on network devices such as routers, switches and access points.

- Congestion management. These tools use packet classification and marking to determine which queue to place the packets in. Congestion management tools include priority queuing; first-in, first-out queuing; and low-latency queuing.

- Congestion avoidance. These tools monitor network traffic for congestion and drop low-priority packets when congestion occurs. Congestion avoidance tools include weighted random early detection and random early detection.

- Shaping. These tools manipulate traffic flowing into the network and prioritize real-time applications over less time-sensitive applications, such as email and messaging. Traffic shaping tools include buffers, generic traffic shaping and frame-relay traffic shaping.

- Link efficiency. These tools maximize bandwidth use and reduce delay for packets accessing the network. While not exclusively for QoS, link efficiency tools are used in conjunction with other QoS mechanisms. Link efficiency tools include Real-time Transport Protocol, header compression, TCP, header compression and link compression.

QoS tools

QoS tools generally fall into these categories:

- Classification. Identifies traffic and marks it to make sure that other network devices can identify and prioritize it.

- Queueing. Reserves bandwidth to hold packets in a buffer to process later.

- Policing. Enforces a specific bandwidth and limit as well as drops packets that don't adhere to the rule. This is part of congestion avoidance.

- Shaping. Similar to policing but queues the excess traffic in a buffer instead of completely dropping them. This, along with queueing, is part of congestion management.

- Weighted random early discard. Drops low-priority data flows to protect high-priority data from the negative effects of network congestion.

- Fragmentation and compression. Lowers bandwidth on a network to prevent delay and jitter.

QoS tools may perform just one or a combination of these functions. Many QoS tools are managed and perform these functions automatically, giving administrators access to alter settings and policies as needed. Some popular QoS monitoring tools include the following:

- SolarWinds NetFlow Traffic Analyzer.

- Paessler PRTG Network Monitor.

- Nagios Network Analyzer.

- ManageEngine NetFlow Analyzer.

- OpenNMS (open source).

- Ntopng (open source).

- BandwidthD (open source).

CHAPTER SIX

SOFTWARE RELIABILITY

Software reliability is the probability of failure-free operation of a computer program for a specified period in a specified environment. Reliability is a customer-oriented view of software quality. It relates to operation rather than design of the program, and hence it is dynamic rather than static.

Software Reliability Specification

The Software requirements specification is the criterion against which reliability is measured. Reliability applies to a specified period of time. In practical terms, this means that a system has a specified chance that it will operate without failure before time.

Software Reliability Metrics

Mean Time to Failure (MTTF)

MTTF examines the amount of time between two occurrences of failure in software usage under normal operating conditions. The metric is an average of the time elapsed between two failures over the total number of failures.

Software failures are almost unavoidable, but this number attempts to quantify how long the software can operate without experiencing a failure. However, it doesn't look at the time it takes to fix the error and get the software running again.

This is a vital, safety-critical metric because it helps developers understand how long a software can perform before failing. It also helps predict failures going forward.

Mean Time to Repair (MTTR)

When software fails, it takes time to fix the error. MTTR measures the average time it takes to track the cause and repair the software fault.

Developers use this metric to understand the length of time needed to fix an error once a failure occurs. Teams look to this number to better understand their working process for reliability and find ways to improve it.

Rate of Occurrence of Failure (ROCOF)

ROCOF is the number of failures that appear in a given time interval (i.e., the number of unexpected events over a specific time of operation).

This helps developers understand the frequency of failure occurrence. While other metrics measure lengths of time between events, ROCOF measures how often the event happens. The value is a ratio of the total amount of failures per the length of time they happen in.

Mean Time Between Failure (MTBF)

MTBF uses previous observations and data to figure out the average time between failures. The number is derived by adding together the MTTF and MTTR metrics.

MTBF values are used to calculate overall failure rates for both repairable and non-repairable products.

For example, an MTBF of X denotes that once a failure occurs, the next failure is expected to appear sometime after X hours have passed.

Probability of Failure on Demand (POFOD)

POFOD refers to the probability that the software will fail when a certain action happens. This can be used in systems where actions or services are requested at an infrequent pace.

It doesn’t involve real-time measurements like other attributes, but it gives developers a sense of how often a failure can be expected versus how many times a request will be successful in a product’s life cycle.

Availability (AVAIL)

AVAIL of software measures the possibility of availability of the system for use over a specified period of time. It calculates the number of failures that happen during a specific period and considers the length of downtime that results when a failure happens.

This is an important metric for software that causes major effects or damage when outages occur, such as telecommunication and operating systems.

Knowing the total time that software could be unavailable over a specific time frame helps teams understand what the repercussions might be.

Software Fault Avoidance and Fault Tolerance

Fault Avoidance is used either minimize the possibility of mistakes and/or that trap mistakes before they result in the introduction of system fault Examples of such techniques include avoiding error-prone programming language constructs such as pointers.

Fault tolerance is the ability of a system to maintain proper operation in the event of failures or faults in one or more of its components. If its operating quality decreases at all, the decrease is proportional to the severity of the failure, as compared to a naively designed system, in which even a small failure can lead to total breakdown.

Fault detection: The development process is organized so that faults in the software are detected and repaired before delivery to the customer.

A fault-tolerant design enables a system to continue its intended operation, possibly at a reduced level, rather than failing completely, when some part of the system fails.

Fault Tolerance: Motivations

• We cannot achieve complete software reliability

• Demonstrating high reliability for safety critical applications is difficult

• How can we ensure an acceptable behavior of the system when failures occur?

• E.g., the computers of an air traffic control systems must be continuously available.

Aspects of Fault Tolerance

• Failure detection: The system must detect that a particular state combination has resulted or will result in a system failure

• Damage assessment: the parts of the system state which have been affected by the failure must be detected.

• Fault recovery: The system must restore its state to a known “safe” state.

• Fault repair: This involves modifying the system so that the fault does not recur. For systems that need to be continuously available, replacing the faulty component is more complex.

Two Main Approaches

• Fault-tolerant architectures: Explicit support for fault tolerance (problem detection, recovery)

• Defensive Programming: No specific architecture. But redundant code to check system state after modification. If inconsistencies are detected, state is restored to a known correct state.

Hardware Reliability

Fault Tolerant Software architectures

• The success of TMR at providing fault tolerance is based on two fundamental assumptions

– The hardware components do not include common design faults

– Components fail randomly and there is a low probability of simultaneous component failure

• Neither of these assumptions are true for software

– It isn’t possible simply to replicate the same component as they would have common design faults

– Simultaneous component failure is therefore virtually inevitable

• Software systems must therefore be diverse

Software Analogies to TMR

• N-version programming

– The same specification is implemented in a number of different versions by different teams. All versions compute simultaneously and the majority output is selected using a voting system..

– This is the most commonly used approach e.g. in Airbus 320.

• Recovery blocks

– A number of explicitly different versions of the same specification are written and executed in sequence

– An acceptance test is used to select the output to be transmitted.

N-version Programming

• Using a common specification, the software system is implemented in a number of different versions by different teams

• Versions are executed in parallel

• Outputs are compared using a voting system and inconsistent outputs are rejected

• At least three versions should be available

• Assumption: it is unlikely different teams will make the same design or programming errors

• However, there is some empirical evidence that teams commonly misinterpret specifications in the same way and use the same / similar algorithms in their systems.

N-version Programming

Recovery Blocks

• Finer grain approach to fault tolerance

• Each program component includes a test to check if the component has executed successfully

• It includes alternative code to back-up and repeat the computation with another algorithm (versions) if the test detects a failure

• Versions are executed in sequence.

• The output which conforms to an “acceptance test” is selected.

• Reduce probability of common errors as different algorithms MUST be used for each recovery block

• The weakness in this system is writing an appropriate acceptance test.

Software Safety and Hazard Analysis

Software system safety optimizes system safety in the design, development, use, and maintenance of software systems and their integration with safety-critical hardware systems in an operational environment.

Software hazard analysis is a form of subsystem hazard analysis. It validates that specified software blackbox behavior satisfies system safety design constraints. Software Hazard Analysis checks that specified software behavior satisfies general software system safety design criteria, as well.

Hardware Reliability vs Software Reliability

| Features | Hardware Reliability | Software Reliability |
| --- | --- | --- |
| Source of Failure | Failures are caused due to defects in design, production, and maintenance. | Failures are caused due to defects in design. |
| Wear and Tear | Failure occurs due to physical deterioration in wear and tear. | In software reliability, there is no wear and tear. |
| Deterioration Warning | In this prior deterioration warning about failure. | In this no prior deterioration warning about failure. |
| Failure Curve | The bathtub curve is used for failure rates apply. | There is no Bathtub curve for failure rates. |
| Is Failure Time-dependent? | In this failures are time-dependent. | In this failures are not time-dependent. |
| Reliability Prediction | In this reliability can be predicted from design. | In this reliability can not be predicted from design. |
| Reliability Complexity | The complexity of hardware reliability is very high. | The complexity of software reliability is low. |
| External Environment Impact | Hardware reliability is related to environmental conditions. | External environment conditions do not affect software reliability. |
| Reliability Improvement | Reliability can’t be improved through redundant of hardware. | Reliability can be improved through redundancy of software. |
| Maintenance | Repairs can be made that make hardware more reliable through maintenance. | No equivalent preventive maintenance for software. |

Software Quality Assurance (SQA) is simply a way to assure quality in the software. It is the set of activities which ensure processes, procedures as well as standards are suitable for the project and implemented correctly.

Elements Of Software Quality Assurance:

- Standards: The IEEE, ISO, and other standards organizations have produced a broad array of software engineering standards and related documents. The job of SQA is to ensure that standards that have been adopted are followed, and all work products conform to them.

- Reviews and audits: Technical reviews are a quality control activity performed by software engineers for software engineers. Their intent is to uncover errors. Audits are a type of review performed by SQA personnel (people employed in an organization) with the intent of ensuring that quality guidelines are being followed for software engineering work.

- Testing: Software testing is a quality control function that has one primary goal—to find errors. The job of SQA is to ensure that testing is properly planned and efficiently conducted for primary goal of software.

- Error/defect collection and analysis: SQA collects and analyzes error and defect data to better understand how errors are introduced and what software engineering activities are best suited to eliminating them.

- Change management: SQA ensures that adequate change management practices have been instituted.

- Education: Every software organization wants to improve its software engineering practices. A key contributor to improvement is education of software engineers, their managers, and other stakeholders. The SQA organization takes the lead in software process improvement which is key proponent and sponsor of educational programs.

- Security management: SQA ensures that appropriate process and technology are used to achieve software security.

- Safety: SQA may be responsible for assessing the impact of software failure and for initiating those steps required to reduce risk.

- Risk management: The SQA organization ensures that risk management activities are properly conducted and that risk-related contingency plans have been established.

Major Software Quality Assurance Activities:

- SQA Management Plan: Make a plan for how you will carry out the SQA throughout the project. Think about which set of software engineering activities are the best for project. check level of SQA team skills.

- Set The Check Points: SQA team should set checkpoints. Evaluate the performance of the project on the basis of collected data on different check points.

- Measure Change Impact: The changes for making the correction of an error sometimes re introduces more errors keep the measure of impact of change on project. Reset the new change to check the compatibility of this fix with whole project.

- Multi testing Strategy: Do not depend on a single testing approach. When you have a lot of testing approaches available use them.

- Manage Good Relations: In the working environment managing good relations with other teams involved in the project development is mandatory. Bad relation of SQA team with programmers team will impact directly and badly on project. Don’t play politics.

- Managing Reports and Records: Document and share QA activities (test cases, defects, client changes) for future reference and stakeholder alignment.

Benefits of Software Quality Assurance (SQA):

- SQA produces high quality software.

- High quality application saves time and cost.

- SQA is beneficial for better reliability.

- SQA is beneficial in the condition of no maintenance for a long time.

- High quality commercial software increase market share of company.

- Improving the process of creating software.

- Improves the quality of the software.

- It cuts maintenance costs. Get the release right the first time, and your company can forget about it and move on to the next big thing. Release a product with chronic issues, and your business bogs down in a costly, time-consuming, never-ending cycle of repairs.

Disadvantage of SQA:

There are a number of disadvantages of quality assurance. Some of them include adding more resources, employing more workers to help maintain quality and so much more.

Software quality metrics are a subset of software metrics that focus on the quality aspects of the product, process, and project. These are more closely associated with process and product metrics than with project metrics.

Software quality metrics can be further divided into three categories

- Product quality metrics

- In-process quality metrics

- Maintenance quality metrics

Formal Technical Review (FTR) is a software quality control activity performed by software engineers.

Objectives of formal technical review (FTR):

- Useful to uncover errors in logic, function, and implementation for any representation of the software.

- The purpose of FTR is to verify that the software meets specified requirements.

- To ensure that software is represented according to predefined standards.

- It helps to review the uniformity in software that is developed in a uniform manner.

- To make the project more manageable.

Statistical Quality Assurance

SQA is used to identify the potential variations in the manufacturing process and predict potential defects on a parts-per-million (PPM) basis. It provides a statistical description of the final product and addresses quality and safety issues that arise during manufacturing.

What Is An ISO 9000?

ISO 9000 is a series of international standards concerning quality management and assurance developed to help organizations establish and maintain an effective quality management system (QMS). The standards can help business entities fulfill regulatory requirements, improve continuously, and ensure customer satisfaction.

ISO 9000 series lays out the principles, best practices, and guidelines for quality management systems. It leads to improved documentation of organizational processes, making the production process more efficient, reducing wastage, and minimizing costs. The standards are not specific to any particular industry and can apply to companies irrespective of their size.

ISO 9000 Certification Explained

ISO 9000 meaning refers to a set of globally-accepted standards introduced by the International Organization for Standardization (ISO) to help organizations establish, maintain and continuously improve their QMSs. An ISO certification enables businesses to provide their customers with the best products and services. Therefore, one should consider it the base level of any quality system.

ISO published these standards for both profit and non-profit organizations. The series covers the fundamentals and vocabulary of an ideal QMS. Moreover, it describes the seven quality management requirements or principles.

A business can get ISO certification only after a third party carries out a successful audit. All organizations must fulfill the following requirements to clear the audit and obtain this certificate:

- Companies must develop and implement a QMS that aligns with the principles of the relevant series of standards.

- Organizations must get an audit performed by a certified body to evaluate their QMS’s performance against the set standard.

Once businesses obtain the certificate, they have to recertify it every three years to ensure that they are up to date with the latest standards.

ISO 9000 Standards

The series comprises the following standards:

- ISO 9000:2015: This standard defines the specifics of QMSs while clearly stating the terms utilized in such standards.

- ISO 19011:2018: It offers guidance regarding internal audits, which confirm and increase the efficacy of a QMS and external audits.

- ISO 9004:2018: This standard offers guidance for continuous success per the seven quality management principles or requirements. It specifies the principles that the senior members of the company’s management team must utilize to boost performance while considering shareholders’ requirements.

- ISO 9004:2018: It offers companies guidance on fulfilling the relevant regulatory and quality requirements while increasing shareholders’ satisfaction. Moreover, it offers guidance to organizations on how to have an effective technique for spotting and implementing improvements.

ISO 9000 Requirements

Let us look at the ISO 9000 series requirements.

1. Customer Focus

Organizations must understand the requirements of existing and future customers and aim to exceed their expectations. They can ask for feedback from customers and monitor complaints.

2. Engagement

Everyone at every level of an organization should improve the business’s capability to create customer value.

3. Leadership

A company’s leaders should establish a unity of direction throughout the organization to equip and empower the employees to achieve quality-related objectives.

4. Process

Organizations must manage their activities as interrelated processes that work as a well-organized system. Moreover, they must utilize process analysis tools to measure their capability and spot relations between activities to improve the processes.

5. Evidence-based Decision Making

Companies must make all decisions after thoroughly analyzing and evaluating the necessary data and information for the best results. One must remember to balance data analysis with qualitative evidence and practical experience.

6. Relationship Management

For short-term and long-term success, business entities must manage their relationships with related parties, such as vendors, partners, and contractors.

7. Continuous Improvement

Organizations must adopt an approach that focuses on continuous improvement; they should empower workers to make improvements and celebrate the same. Moreover, they must measure the improvements consistently.

Example

In May 2022, a popular EV technology company, Cenntro Electric Group Limited, announced that it would take over a new production facility in Changxing for roughly $19.5 million. This new acquisition will significantly improve the company’s production capacity. Moreover, it will help Cenntro obtain ISO 9000 certification enabling the company to establish and maintain an effective QMS. This, in turn, can improve business efficiency and ensure customer satisfaction.

Trial production will begin from Q3, 2022, per expectations.

Benefits

ISO 9000 benefits businesses, their employees, and customers. Let us look at the advantages.

- A business becomes well-equipped to address feedback efficiently and quickly. Resultantly, customer satisfaction increases.

- ISO certification serves as a badge of reliability and improves brand image. It helps in gaining customer trust.

- The series of standards encourages preventive thinking, which enables businesses to address issues before they arise and impact customers.

- It minimizes the errors in recurring processes.

- The standards help businesses save time through the efficient management of resources.

- The cumulative effects of implementing these standards can result in increased market share and profit potential.

- The set standards reduce manual work through process automation and process integration.

- Job security increases as these standards improve business performance.

- Employees have a clear idea regarding their roles and responsibilities. This improves job satisfaction.

- Employees feel more responsible for their jobs owing to accountability and process ownership. As a result, process quality improves.

1. Why ISO 9000 is important?

- The series aims to implement and maintain a QMS that can increase productivity and minimize costs within an organization while fulfilling regulatory requirements and ensuring customer satisfaction.

2. What are the elements of ISO 9000?

- There are 20 elements in the series. Some of them are as follows:
– Inspection and testing
– Product availability and traceability
– Design control
– Quality system
– Management responsibility
– Internal quality audits
– Servicing
– Training
– Contract review
– Process control
–Purchasing
– Document and data control

3. How much does ISO 9000 certification cost?

- The certification cost depends on various factors, for example, the complexity and size of the organization, scope of registration, etc. The expenses can range from $12,000-50,000, in addition to the cost of registration audits for implementing a formal management system. Note that this is only a rough estimate.

4. What is ISO 9000 vs 9001?

- ISO 9000 is a set or series of quality management standards, and ISO 9001 is a standard within that series.